EE-CE 201 - Linear Circuit Analysis I

Fall 2025

Project Deliverable 1: Technical Report

Battery Range Indicator

| Group number | Section Number | IDS | Name |
| --- | --- | --- | --- |
| 9 | U1 | 75018 | Yousef AlAbdulrazzaq |
|  |  | 57078 | Yousef AlAbkal |
|  |  | 10086967 | Essa Altaher |

Contents:

- Project requirements

(Provide a brief description of the requirements showing the group-dependent calculations using the student IDs)

- Test 1

It should contain the following parts:

  - Objective of the test and simulated scenario

  - Explanation of what was applied in the circuit to perform it

  - Results illustrated with figures, graphs, etc.

  - Results explanation, analysis and discussion

- Test 2

It should contain the following parts:

  - Objective of the test and simulated scenario

  - Explanation of what was applied in the circuit to perform it

  - Results illustrated with figures, graphs, etc.

  - Results explanation, analysis and discussion

.

.

.

- Test N

It should contain the following parts:

  - Objective of the test and simulated scenario

  - Explanation of what was applied in the circuit to perform it

  - Results illustrated with figures, graphs, etc.

  - Results explanation, analysis and discussion

Important note: tests should cover all the possible operation cases for the circuit as shown in the project description. The document should contain as many tests as necessary for this purpose.

Project Brief:

We have been employed to design a system that measure a battery’s voltage 𝑉𝑏𝑎𝑡 and display the charge status of the battery using three LEDs.

A 9V battery is used for these tests therefore Vmax=9.5V and Vmin=7.25V

- Green LED: Battery is in the normal range, 𝑉𝑡ℎ1 < 𝑉𝑏𝑎𝑡 < 𝑉𝑡ℎ2

- Red LED: Battery is in the critical threshold, 𝑉𝑏𝑎𝑡 > 𝑉𝑡ℎ2

- Yellow LED: Battery is low on charge, 𝑉𝑏𝑎𝑡 < 𝑉𝑡ℎ1

The thresholds are calculated using the equation:

Vth1= 7+5+0+1+8 +5+7+0+7+8 + (1+0+0+8+6+9+6+7)10 + 7+5+0+1+8+5+7+0+7+8+1+0+0+8+6+9+6+7*Vmax

Vth2=Vmin+ Vth12

Therefore:

Vth1= 8.5V

Vth2= 11.48V

We chose a window comparator design using operational amplifiers, diodes and transistor LED drivers. This is a reliable design and consists of components all rated for a single 12V supply.

Complete Circuit Schematic: (9V)

Circuit Description:

Power Supply: BAT1 is a 12V supply connected to VCC_CIRCLE which supplies power to all active components (op-amps, Logic gates etc.)

Battery Input: BAT2 is the test battery that is being measured.

Reference Voltages:

Vref1 =  VCC​×R2R1+R2​=12V×10kΩ430Ω+10kΩ​=11.51V≈11.48V

Vref2 =  VCC​×R4R3+R4​=12V×6.8kΩ3kΩ+6.8kΩ​=8.33V≈8.5V

R1 (430Ω) and R2 (10kΩ) - Vref1 Divider: Form a voltage divider creating Vref1, the high threshold voltage.

R3 (3kΩ) and R4 (6.8kΩ) - Vref2 Divider: Form a voltage divider creating Vref2, the low threshold voltage

OPAMPS:

U1A: When BAT2 (9V) > Vref2 (8.327V), output goes HIGH, indicating battery is not depleted.

U1B: When Vref1 (11.51V) > BAT2 (9V), output goes HIGH, indicating battery is not overcharged

Diodes:

D1: When Above-Min is LOW, D1 conducts and pulls Green-OK LOW

D2: When Below-Max is LOW, D2 conducts and pulls Green-OK LOW

R5 (10kΩ) - Green-OK Pull-Up Resistor: Only when BOTH Above-Min AND Below-Max are HIGH do both diodes block, allowing R5 to pull Green-OK HIGH

Green-OK Node: The AND gate output - HIGH only when battery is in normal range

LED Driver Circuits:

RED:

Q4 (PNP Transistor): Turns ON when base is LOW. When Vbat > 11.48V, Below-Max goes LOW, turning Q4 ON

R12 (1kΩ): Ensures Q4 turns completely OFF when Below-Max is HIGH by bleeding off leakage current

R6 (10kΩ): Limits base current to safe levels when Q4 is conducting

R9 (330Ω): Limits LED current to safe operating levels

YELLOW:

Q5 (PNP Transistor): Turns ON when base is LOW. When Vbat < 8.5V, Above-Min goes LOW, turning Q5 ON

R13 (1kΩ): Ensures Q5 turns completely OFF when Above-Min is HIGH

R10 (10kΩ): Limits base current when Q5 is conducting

R11 (330Ω) - Yellow LED Current Limiting Resistor: Limits LED current to safe operating levels

GREEN:

Q3 (NPN Transistor): Turns ON when base is HIGH. When Green-OK is HIGH Q3 conducts

R7 (10kΩ): Limits base current to Q3

R8 (330Ω): Limits LED current to safe operating levels

Test 1: Normal Battery (9V)

This test verifies that the Green LED illuminates when the battery voltage is in its normal operating range.

This test simulates a fresh battery in normal condition:

- Battery voltage is above minimum safe threshold (> 8.5V)

- Battery voltage is below maximum safe threshold (< 11.48V)

- System should indicate "normal" status with Green LED active

Simulation conditions: BAT2 is set to 9V.

Expected Results:

| LED | Expected State | Reason |
| --- | --- | --- |
| Red (D1) | OFF | BAT2 (9V) > Vth2 X |
| Green (D2) | ON | Vth1 < BAT2 < Vth2 ✓ |
| Yellow (D3) | OFF | BAT2 < Vth1 X |

Experimental Results:

| Component | Measured Value | Explanation |
| --- | --- | --- |
| Test Battery (BAT2) | 9V | Battery under test |
| Vref1 | 11.51V | High threshold from R1/R2 divider |
| Vref2 | 8.33V | Low threshold from R3/R4 divider |
| Above-Min | 11.11V (HIGH) | 9V > 8.33V → output HIGH |
| Below-Max | 11.11V (HIGH) | 11.51V > 9V → output HIGH |
| Green-OK | 10.29V (HIGH) | Both diodes reverse-biased, pulled up by R5 |
| Q4 Base | 11.92V | Close to VCC, PNP OFF |
| Q5 Base | 11.92V | Close to VCC, PNP OFF |
| Q3 Base | 778mV | Above 0.7V threshold, NPN ON |
| RED LED (D4) | 2.058mV | OFF - No current flow |
| YELLOW LED (D5) | 2.058mV | OFF - No current flow |
| GREEN LED (D3) | ~702 mV | ON - Current flowing |

Result: GREEN LED ON, RED & YELLOW LEDs OFF – Battery is in normal range.

Test 2: Critical Battery (12V)

This test measures a battery that above the critical threshold i.e. an overcharged battery.

This test simulates a 9V battery in critical condition:

- Battery voltage is above maximum safe threshold (> 11.48V)

- System should indicate "critical" status with Red LED active

Simulation conditions: BAT2 is set to 12V

Expected Results:

| LED | Expected State | Reason |
| --- | --- | --- |
| Red (D1) | ON | BAT2 (12V) > Vth2 ✓ |
| Green (D2) | OFF | Vth1 < BAT2 < Vth2 X |
| Yellow (D3) | OFF | BAT2 < Vth1 X |

Experimental Results:

| Component | Measured Value | Explanation |
| --- | --- | --- |
| Test Battery (BAT2) | 12V | Battery under test |
| Vref1 | 11.51V | High threshold from R1/R2 divider |
| Vref2 | 8.33V | Low threshold from R3/R4 divider |
| Above-Min | 11.11V (HIGH) | 12V > 8.327V → output HIGH |
| Below-Max | 36.83mV (LOW) | 11.51V < 12V → output LOW |
| Green-OK | ~0.7V (LOW) | D2 pulls LOW |
| Q4 Base | 11.19V | Below-Max LOW pulls base lower than VCC |
| Q5 Base | 11.92V | Above-Min HIGH keeps base close to VCC |
| Q3 Base | 688.2mV | Near threshold |
| RED LED (D4) | 701.9mV | ON - Current flowing through LED |
| YELLOW LED (D5) | 2.058mV | OFF - No current flow |
| GREEN LED (D3) | 22.67mV | OFF - Minimal current |

Result: Red LED ON (701.9mV), Yellow LED OFF, Green LED OFF - Overcharge warning.

Test 3: Depleted Battery (7V)

This test simulates a battery that has been discharged through normal use and requires recharging.

This test simulates a 7V battery:

- Battery voltage is below the lower threshold (< 8.5V)

- System should indicate "Low" status with Yellow LED active

Simulation conditions: BAT2 is set to 7V

Expected Results:

| LED | Expected State | Reason |
| --- | --- | --- |
| Red (D1) | OFF | BAT2 (7V) > Vth2 X |
| Green (D2) | OFF | Vth1 < BAT2 < Vth2 X |
| Yellow (D3) | ON | BAT2 < Vth1 ✓ |

Experimental Results:

| Component | Measured Value | Explanation |
| --- | --- | --- |
| Test Battery (BAT2) | 7V | Battery under test |
| Vref1 | 11.51V | High threshold from R1/R2 divider |
| Vref2 | 8.33V | Low threshold from R3/R4 divider |
| Above-Min | 36.71mV (LOW) | 7V < 8.33V → output LOW |
| Below-Max | 11.11V (HIGH) | 11.51V > 7V → output HIGH |
| Green-OK | ~0.7V | D1 pulls LOW due to Above-Min LOW |
| Q4 Base | 11.92V | Below-Max HIGH keeps base close to VCC |
| Q5 Base | 11.19V | Above-Min LOW pulls base lower |
| Q3 Base | 688.2mV | Near threshold |
| RED LED (D4) | 2.058mV | OFF - No current flow |
| YELLOW LED (D5) | 701.9mV | ON - Current flowing through LED |
| GREEN LED (D3) | 22.67mV | OFF - Minimal current |

Results: Yellow LED ON (701.9mV), Red LED OFF, Green LED OFF - Low battery warning.

Limitations of the simulation:

We could not use the actual LEDs in the simulation as ORCAD CAPTURE CIS Lite doesn’t have working models available for use and simulation, so we used regular diodes that work at 700mV, not the regular 1-3V that LEDs use.